Process Documentation

16.11.2025

- creating a GitHub repository

- Setting up a README file

- Setting up a Process Documentation file

30.11.2025

-creating a To-Do list:

-Lights

-Servo

-Sound

-making it Pretty

Laying out all the objects I would consider using:

Why I didn’t use..

Battery: I connected a lot of devices and let them run while coding and I didn’t want my battery to run out during the trials

Distance sensor: this was an optional for me from the beginning, and I had two issues, first of all there was already a lot of pin compilations and I didn’t want to risk another one. Second I didn’t really see how I could include it in my creative packaging without making it stand out.

Lights:

- plugging in all lights

- Plugging in all grounds

- plugging in all resistors

- testing all lights

- coding the code to make them flicker at random like a stormy cloud

  - set-up for the pins

  - working with random so that the LEDs seem to be lighting up in a natural manner

  - the LEDs slowly light up and dim down in a random time frame with a random delay

Music:

- connecting the buzzer

- writing the music code

- adding a pitches.h file

- finding a Christmas song online

- had to rewire, cause pin 3 and 11 were colliding with the sound

- fixing this issue in code: [Video 1]

- fixed it by rewiring

- set-up the buzzer and making the music play separated from the lights

Servo:

- got the servo wired and working but still complications between Servo and lights

- A lot of rewiring and trial and error, because the Servo lagged -> I suspect that the Servo tried to follow the clock of the LEDs

- Okay, tried everything and concluded that I can only use 6 and 7 for my LEDs, sadly only have to different rhythms for my light cloud now.

My final Set up:

Pretty:

- Having to rearrange the lightbulbs, because the greens ones are a lot duller

- Putting cotton over the LEDs to make it look like a cloud

Sources:

- The official Arduino Docs

- The slides from class

- My notes from class

- https://www.hibit.dev/posts/31/play-christmas-melody-with-arduino-and-a-buzzer ( for the song)

- A lot of reddit and pages alike